| ОПИСАНИЕ НА ПРЕДМЕТА НА ДОСТАВКАТА (ТЕХНИЧЕСКА СПЕЦИФИКАЦИЯ) I. Предмет на доставкатa: 1. Комплектно оборудване за производство на стенни кухотелни панели (спироли) – 1 комплект II. Функционално предназначение: Комплектно оборудване за производство на стенни кухотелни панели (спироли), състоящо се от (1) Дюзен модул за производство на стенни елементи с височина h = 120 мм и (2) Сменяеми части 2х600 мм за стенни елементи към дюзения модул. Комплектното оборудване представлява допълнителна окомплектовка към вече внедрен в предприятието екструдер, предназначен за производството на бетонови кухотелни панели (спироли) с приложение в строителството. III. Технически и функционални характеристики и комплектация: Комплектното оборудване за производство на стенни кухотелни панели (спироли), включва: (1) Дюзен модул за производство на стенни елементи h = 120 мм: - Предназначен за леене на кухотелни панели (спироли) с 8 кухини и с номинална дебелина от 180 мм върху леярска основа; - Представлява модул към екструдерна машина; - Основни компоненти на дюзовия модул са: рамка, дюза, монтаж на страничната плоча, водач на нишки и нивелираща греда; - Рамката е с две надлъжни греди, с колела; - Дюзата е ядрото на дюзовия модул. Състои се от: - Скоростна кутия; - Въртящи се оси за захранващи шнекове; - Захранващ винт и ръкави; - Плоча за маса и водачи за маса; - Вътрешна дърпаща ос за задържане на ръкава на място; - Напречен колянов вал, свързващи пръти за задвижване на движението на уплътняване; - Задвижваща система за накланяне на ръкава; - Страничните плочи се състоят от основна рамка и сменяема износваща се плоча; - Нивелиращите греди се състоят от рамка, мотор-редуктор, ос и щанги за преместване на нивелиращата плоча, нивелираща плоча и изглаждаща плоча; - Система за смазване; - Водач за нишки, управляван механично; - Технически данни: - Дължина: 4350 мм - Ширина: 1670 мм - Височина: 845 мм - Междурелсие, стандартно: 1290 мм - Междурелсие, регулируемо: 1260 - 1320 мм - Предпазващо от ръжда покритие върху небоядисаните повърхности; (2) Сменяеми части 2х600 мм за стенни елементи - Сменяемите части за дюзения модул са предназначени да променят модула, така че да произвежда кухотелни панели с различно напречно сечение; - Комплектът съдържа необходимите части за едно напречно сечение на плоча; - Сменяемите части за дюзения модул за производство на сплит плоча с височина на плочата от 120-200 мм и вкючва: - Нивелираща греда; - Средна странична форма; - Два тесни ръкава; - Предпазващо от ръжда покритие върху небоядисани повърхности; | DESCRIPTION OF THE SUBJECT OF DELIVERY (TECHNICAL SPECIFICATION) I. Subject of delivery: 1. Complete equipment for the production of wall hollow panels (spirolis) – 1 set II. Functional purpose: Complete equipment for the production of wall hollow panels (spirolis), consisting of (1) Nozzle module for the production of wall elements with a height of h = 120 mm and (2) Replaceable parts 2x600 mm for wall elements to the nozzle module. The complete equipment is an additional equipment to the extruder already implemented in the enterprise, intended for the production of concrete hollow panels (spirolis) with application in construction. III. Technical and functional characteristics and equipment: Complete equipment for the production of wall hollow panels (spirolis), includes: (1) Nozzle module for the production of wall elements with a height of h = 120 mm: - Designed for casting hollow panels (spirolis) with 8 cavities and a nominal thickness of 180 mm on a casting base; - Represents a module for an extruder machine; - Main components of the nozzle module are: frame, nozzle, side plate assembly, filament guide and leveling beam; - The frame has two longitudinal beams, with wheels; - The nozzle is the core of the nozzle module. Consists of: - Gearbox; - Rotating axes for feed augers; - Feed screw and sleeves; - Table plate and table guides; - Internal pull shaft to hold the sleeve in place; - Cross crankshaft, connecting rods to drive the compaction movement; - Drive system for tilting the sleeve; - The side plates consist of a main frame and a replaceable wear plate; - The leveling beams consist of a frame, a gear motor, an axle and rods for moving the leveling plate, leveling plate and smoothing plate; - Lubrication system; - Mechanically controlled thread guide; - Technical data: - Length: 4350 mm - Width: 1670 mm - Height: 845 mm - Track gauge, standard: 1290 mm - Track gauge, adjustable: 1260 - 1320 mm - Rust-preventive coating on unpainted surfaces; (2) Replaceable parts 2x600 mm for wall elements - The replaceable parts for the nozzle module are designed to modify the module to produce hollow core panels with different cross-sections; - The kit contains the necessary parts for one slab cross-section; - The replaceable parts for the nozzle module for the production of a split slab with a slab height of 120-200 mm and include: - Leveling beam; - Middle side form; - Two narrow sleeves; - Rust-preventive coating on unpainted surfaces; |
| --- | --- |